1.  This zip file contain 2 files : EE.EXE and EE.DAT

2.  Please check your LAN-ID No. ( You can find the LAN-ID No. in a white stick label on the board like: EA=XXXXXXXXXXXX)

3.  For example your LAN-ID No. is : EA=1A2A3A4A5A6A

4.  Edit EE.DAT file. (EE.DAT is a text file. You can use WordPad or other software to edit it.)

5.  When you open the EE.DAT file, you can see the following text :

0900    ; Signature

000f     ; Autoload option

1039    ; Vendor ID

0900    ; Device ID

1039    ; Sub-vendor ID

0900    ; Sub-device ID

fe02     ; PMC (1.0a)

ffff

2A1A    ; Mac addr [2][1]

4A3A    ; Mac addr [4][3]

6A5A    ; Mac addr [6][5]

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

ffff

1A 2A 3A FF FF FF    ; Mac addr limit

1A 2A 3A 00 00 00     ; Mac addr start

6.  Please chage the 1A2A3A4A5A6A to your own LAN-ID No.

7.  Save the file.

8.  Enter to the pure DOS mode. Run command : EE EE.DAT

9.  Shutdown the power. ( You have to pull out the power cord or turn off the power supply)

10. Restart your computer