#### 4.2.5.7 	Master WSD shut-down update

##### 4.2.5.7.1	Definition

##### The WSD update is the process by which a WSDB informs a master WSD that its operational parameters, and those of the slave WSDs attached it, are still valid or no longer valid.

##### 4.2.5.7.2	Requirements

A master WSD shall support WSD update function.  For this, a master WSD shall either

- be able to receive an update from the Controller Database within Tping seconds (push update), or

- send an update request to the Controller Database every Tping seconds (pull update).

A master WSD shall support a Tping value of [60] seconds or higher.

A master WSD shall cease transmission, and shall instruct the slaves attached to it to cease transmission, if it receives update from the WSDB that the operational parameters are no longer valid.

A master WSD which has been in sleep mode, i.e. inactive but not powered down, and wants to restart transmissions shall not transmit until:

- if it supports the push update, it has received, within Tping seconds, an instruction from the Controller Database that the operational parameters are still valid, or

- if it supports pull update, it has contacted the Controller Database and it has received an instruction from the Controller Database that the operational parameters are still valid.

A master device that is in sleep mode does not need to be reachable by the Controller Database every Tping seconds, and does not need to contact the Controller Database every Tping seconds.

###### 4.2.5.7.2.1 	Lost connection between the master WSD and the WSDB

4.2.5.7.2.1.1	Definition

A master WSD shall deem that the connection with the WSDB has been lost if

- it is not able to receive an update from the controller database within Tping seconds, if it supports push update, or

- it does not receive an instruction from the controller database within Tping seconds, if it supports pull update.

4.2.5.7.2.1.2	Requirements

A master WSD shall cease transmission, and shall instruct the slaves attached to it to cease transmission, if it loses connection with the WSDB.

###### 4.2.5.7.2.2 	Slave update

4.2.5.7.2.2.1	Definition

The WSD update is the process by which a slave WSD confirms with the serving master WSD that it can continue transmitting according to its current operational parameters.

4.2.5.7.2.2.2	Requirements

A slave WSD shall cease transmission within 1 second when updated to do so by its serving master WSD.

A slave shall cease transmission within five seconds of discovering that it can no longer receive updates from its serving master WSD.

A slave WSD which has been in sleep mode, i.e. inactive but not powered down, and wants to restart transmissions shall,  initially get an update from the serving master WSD confirming that the slave parameters are still valid, before continuing with further transmissions